| 🗹 Preliminary Examination | □ Midterm Examination | □ Final Examination |
| --- | --- | --- |

ITP105- NETWORKING AND COMMUNICATION 2

| MIdyear | Semester, AY | 2025-2026 |
| --- | --- | --- |

| Student Name (LN, FN MI.): | Magbayao, Jonathan D. Melu, John Lloyd E. Palentinos, Althea Kaye T. Reves, Mark Jerome G. Samson, Marco C. |  |  | Date: | 5/21/2026 |
| --- | --- | --- | --- | --- | --- |
| Section: | 3IT-C | Instructor: | Sairine Pregonero | Sem./ A.Y. | Mid/ 25-26 |
| Video Presentation Link: | (Make sure it is accessible) |  |  |  |  |

PRELIM LABORATORY EXAMINATION

VLAN and Switch Security Configuration

Overview

Objective: Configure VLANs, trunking, and basic switch security to establish a functional switched network.

Scenario: A company wants to organize its departments using VLANs and secure the switch ports from unauthorized access. Configure the network according to the given requirements.

Devices Required: 2 Switches, 4 PCs

Devices

PART 1:  BASIC SWITCH CONFIGURATION

Configure the following on BOTH switches:

- Hostname

- Console Password

- Enable Secret Password

- MOTD Banner

- Password Encryption

- Disable DNS Lookup

NOTE: Use the SURNAME OF THE LEADER as all passwords.

Switch 1 Hostname: S1

Switch 2 Hostname: S2

Leader’s Surname (Password): Melu

PART 2: VLAN CONFIGURATION

Create the following VLANs on both switches:

| VLAN ID | VLAN Name |
| --- | --- |
| 10 | HR |
| 20 | SALES |
| 30 | IT |

PART 3 : SWITCH PORT ASSIGNMENT

SWITCH 1:

| Port | Device | VLAN |
| --- | --- | --- |
| Fa0/1 | PC1 | VLAN 10 (HR) |
| Fa0/2 | PC2 | VLAN 20 (SALES) |

SWITCH 2:

| Port | Device | VLAN |
| --- | --- | --- |
| Fa0/1 | PC3 | VLAN 30 (IT) |
| Fa0/2 | PC4 | VLAN 30 (IT) |

PART 4 — TRUNK CONFIGURATION

Configure the connection between Switch 1 and Switch 2 as a TRUNK link.

Trunk Port on S1: _____________

Trunk Port on S2: _____________

Mode: TRUNK

PART 5 : IP ADDRESSING

Each group must create and assign their own valid IP Addressing Scheme.

IP Addressing Table:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | NIC | ______________ | ______________ | ______________ |
| PC2 | NIC | ______________ | ______________ | ______________ |
| PC3 | NIC | ______________ | ______________ | ______________ |
| PC4 | NIC | ______________ | ______________ | ______________ |
| S1 | VLAN 1 | ______________ | ______________ | ______________ |
| S2 | VLAN 1 | ______________ | ______________ | ______________ |

PART 6: PORT SECURITY

Configure Port Security on Fa0/1 of both switches with the following settings:

| Interface | Maximum MAC Address | Violation Mode |
| --- | --- | --- |
| S1 Fa0/1 | 1 | shutdown |
| S2 Fa0/1 | 1 | shutdown |

PART 7: VERIFICATION COMMANDS

Execute the following verification commands and record the results:

| Command | Purpose | Done? |
| --- | --- | --- |
| show vlan brief | Verify VLAN creation and port assignments | ☐ Yes ☐ No |
| show interfaces trunk | Verify trunk link between switches | ☐ Yes ☐ No |
| show running-config | Review all switch configurations | ☐ Yes ☐ No |
| show port-security | Verify port security settings | ☐ Yes ☐ No |
| show ip interface brief | Check IP addressing on switch interfaces | ☐ Yes ☐ No |

PART 8 : CONNECTIVITY TESTING

Perform the following connectivity and verification tests:

| Test | Expected Result | Status |
| --- | --- | --- |
| PC1 pings PC2 (same VLAN 10 — HR) | Both in same subnet, 4 replies expected | ☐ Pass ☐ Fail |
| PC3 pings PC4 (same VLAN 30 — IT) | Both in same subnet, 4 replies expected | ☐ Pass ☐ Fail |
| PC1 pings PC3 (cross-VLAN test) | Should FAIL (different VLANs, no routing) | ☐ Pass ☐ Fail |
| Verify VLAN database on S1 | VLANs 10, 20, 30 visible in show vlan brief | ☐ Pass ☐ Fail |
| Verify trunk link between S1 and S2 | show interfaces trunk shows active trunk | ☐ Pass ☐ Fail |

Observation / Notes:

Performance Rubric:

| Criteria | Excellent (5) | Very Good (4) | Good (3) | Fair (2) | Poor (1) | Score |
| --- | --- | --- | --- | --- | --- | --- |
| Basic Switch Configuration | Hostname, passwords, MOTD, encryption, no DNS — all correct. | Mostly correct, minor errors. | Some configs missing. | Multiple errors. | Failed to configure. |  |
| VLAN Configuration | All VLANs created, named, and functioning correctly. | Minor VLAN errors. | Some VLANs incomplete. | Multiple VLAN errors. | Not configured. |  |
| Port Assignment & Trunk | All ports correctly assigned; trunk functioning. | Minor port/trunk errors. | Some ports incorrect. | Multiple errors. | Failed to configure. |  |
| IP Addressing | All devices have valid IPs, masks, gateways. | Minor IP errors, minimal impact. | Some devices incorrect. | Multiple IP conflicts. | Not configured. |  |
| Port Security | Port security fully and correctly configured. | Minor port security issues. | Partially configured. | Incorrect implementation. | Not configured. |  |
| Verification & Connectivity | All commands executed; all tests successful. | Minor issues in testing. | Some commands incomplete. | Multiple failures. | Not performed. |  |
| Troubleshooting Skills | All problems resolved independently. | Most issues solved, minimal help. | Some issues solved with assistance. | Limited ability shown. | Unable to identify or solve. |  |
| Documentation (PDF) | Complete, organized, all screenshots included. | Mostly complete, minor gaps. | Adequate but lacks organization. | Incomplete and unclear. | Not submitted. |  |
| Video Presentation | Clear, organized, professional; all configs shown. | Understandable, minor issues. | Most tasks covered; lacks clarity. | Unclear or incomplete. | Not submitted or failed. |  |
| TOTAL |  |  |  |  |  | / 45 |

DELIVERABLES

Submit the following:

1. PDF Documentation containing:

• Complete configurations

• Screenshot of VLAN configuration

• Screenshot of trunk configuration

• Screenshot of port security configuration

• Screenshot of verification commands

• Screenshot of successful/failed ping tests

• Network topology

2. Packet Tracer File (.pkt)

3. Video Presentation (5–10 minutes) explaining:

• VLAN creation

• Port assignments

• Trunk configuration

• Port security configuration

• Verification and connectivity testing